- NEEP 602 -- Engineering Problem Solving II

- Exercise 11

- Elliptic Partial Differential Equations

- (Plate Problems)

Here we consider a a particular type of elliptic PDE:  plate theory.  The deformation of plates is governed by the following fourth-order PDE:

¶4w¶x4+2¶2w¶x2¶y2+¶4w¶y4=q(x,y)D

where w is the displacement of the plate, q is the transverse pressure applied to the plate, and

D=Et312(1−υ2)

Here, E is the elastic modulus for the plate material,  is its Poisson’s ratio, and t is the plate thickness.  To difference this equation, it’s convenient to number the mesh points in the neighborhood of a particular point.  I’ll use the following numbering scheme:

5

2

6

10

1

13

11

7

3

12

8

4

9

Our goal is a finite difference approximation to our PDE, written about point 7.  We can write the first term as:

d4wdx4»d2wdx2|3−2d2wdx2|7+d2wdx2|11h2(w1−2w3+w7)−2(w3−2w7+w11)+(w7−2w11+w13)h4w1−4w3+6w7−4w11+w13h4

Similarly,

d4wdx2dy2»d2wdx2|6−2d2wdx2|7+d2wdx2|8h2w2−2w6+w10−2w3−2w7+w11+w4−2w8+w12h4

Substituting these into our PDE yields the following difference equation for the operator:

¶4w¶x4+2¶2w¶x2¶y2+¶4w¶y4»(w1+w5+w9+w13)+2(w2+w4+w10+w12)−8(w3+w6+w8+w11)+20w7h4

This can then be used to solve plate problems.