NEEP 602 -- Engineering Problem Solving II

Homework 2

- Shock System in a Car (20 points)

A vehicle suspension can be modeled as a spring-mass oscillator, as shown below.

x

k

m

y

c

Here m is the mass of the car, k is the stiffness of the suspension system, c is the damping constant for the suspension system, x is the displacement of the car from its equilibrium position, and y is the distance of the road from some datum. We seek the displacement as a function of time for a prescribed road shape.

md2xdt2+cdxdt−dydt+k(x−y)=0The governing equation for a system such as this is:

To complete this problem, we will determine the response of a typical car to the "bump" described by the following equation:

y(t)=0t>λvA1−cos2pvtλt<λv

Use the following inputs:

- k = 60,000 N/m

- m = 900 kg

- c = 4,500 Ns/m

-  = 4 m

- A = 0.04 m

- v=75 miles per hour

# Competing Species (10 points)

The population of a pair of species competing for the same food supply can be modeled as

dxdt=xa−bx−cydydt=yd−ex−fy

Use this model to solve for the populations of x and y if a=0.08, b=0.000001, c=0.000001, d=0.1, e=0.0000001, and f=0.000008. Assume that the initial populations are x=60,000 and y=100,000.

# Radiative Cooling (10 points)

The cooling of a lumped mass by radiation can be modeled by

dTdt+AεσmCT4−Ta4=0

where T is the temperature, A is the surface area of the sphere, m is the mass of the sphere, C is the specific heat,  is the emissivity,  is the Stefan-Boltzman constant, and Ta is the ambient temperature. Solve this system, assuming the density is 8,000 kg/m3, C=500 J/kg-K, =0.5, the initial temperature is 2,500 K, the ambient temperature is 100 K, and the sphere radius is 1 cm.